| Subnet Description | VLAN ID | VLAN Name | Required Number of Hosts | Network Address/CIDR | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Main | VLAN 110 |  | 25 | 10.0.0.160/27 | 10.0.0.161 | 10.0.0.190 | 10.0.0.191 |
|  | VLAN 120 |  | 31 | 10.0.0.64/26 | 10.0.0.65 | 10.0.0.126 | 10.0.0.127 |
| S | VLAN 25 |  | 12 | 10.0.0.224/28 | 10.0.0.225 | 10.0.0.238 | 10.0.0.239 |
|  | VLAN 140 |  | 61 | 10.0.0.0/26 | 10.0.0.1 | 10.0.0.62 | 10.0.0.63 |
| N | VLAN 150 |  | 30 | 10.0.0.128/27 | 10.0.0.129 | 10.0.0.158 | 10.0.0.159 |
|  | VLAN 160 |  | 12 | 10.0.0.240/28 | 10.0.0.241 | 10.0.0.254 | 10.0.0.255 |
| R | VLAN 170 |  | 20 | 10.0.0.192/27 | 10.0.0.193 | 10.0.0.222 | 10.0.0.223 |
|  | VLAN 180 |  | 7 | 10.0.1.0/28 | 10.0.1.1 | 10.0.1.14 | 10.0.1.15 |
| From Main-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.32 /30 | 10.0.1.33 | 10.0.1.34 | 10.0.1.35 |
| From N-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.36/30 | 10.0.1.37 | 10.0.1.38 | 10.0.1.39 |
| From S-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.40/30 | 10.0.1.41 | 10.0.1.42 | 10.0.1.43 |
| From R-MLS to MIU-MIU-GW | - |  | 2 | 10.0.1.44/30 | 10.0.1.45 | 10.0.1.46 | 10.0.1.47 |
| From SW-S to MIU-MIU-GW | - |  | 7 | 10.0.1.16/28 | 10.0.1.17 | 10.0.1.30 | 10.0.1.31 |

Addressing Table

| Device | Interface | Address | Subnet Mask | Default Gateway | VLAN ID |
| --- | --- | --- | --- | --- | --- |
| Main-MLS | G1/1/1 | 10.0.1.33 | 255.255.255.252 | 10.0.1.34 | N/A |
|  | VLAN 110 | 10.0.0.161 | 255.255.255.224 | 10.0.1.34 | VLAN 110 |
|  | VLAN 120 | 10.0.0.65 | 255.255.255.192 | 10.0.1.34 | VLAN 120 |
| S-MLS | G1/1/1 | 10.0.1.41 | 255.255.255.252 | 10.0.1.42 |  |
|  | VLAN 25 | 10.0.0.225 | 255.255.255.240 | 10.0.1.42 | VLAN 25 |
|  | VLAN 140 | 10.0.0.1 | 255.255.255.192 | 10.0.1.42 | VLAN 140 |
| N-MLS | G1/1/1 | 10.0.1.37 | 255.255.255.252 | 10.0.1.38 |  |
|  | VLAN 150 | 10.0.0.129 | 255.255.255.224 | 10.0.1.38 | VLAN 150 |
|  | VLAN 160 | 10.0.0.241 | 255.255.255.240 | 10.0.1.38 | VLAN 160 |
| R-MLS | G1/0/5 | 10.0.1.45 | 255.255.255.252 | 10.0.1.46 |  |
|  | VLAN 170 | 10.0.0.193 | 255.255.255.224 | 10.0.1.46 | VLAN 170 |
|  | VLAN 180 | 10.0.1.1 | 255.255.255.240 | 10.0.1.46 | VLAN 180 |
| MIU-GW | Gig0/0 | 209.165.200.226 | 255.255.255.240 | N/A |  |
|  | Gig0/1 | 10.0.1.46 | 255.255.255.252 | N/A |  |
|  | Gig0/0/0 | 10.0.1.17 | 255.255.255.240 | N/A |  |
|  | Gig0/1/0 | 10.0.1.34 | 255.255.255.252 | N/A |  |
|  | Gig0/2/0 | 10.0.1.42 | 255.255.255.252 | N/A |  |
|  | Gig0/3/0 | 10.0.1.38 | 255.255.255.252 | N/A |  |
| ISP | Gig 0/0 | 209.165.200.225 | 255.255.255.240 | N/A |  |
|  | Gig 0/1 | 64.100.1.1 | 255.255.255.224 | N/A |  |
|  | Gig 0/2 | 64.100.2.1 | 255.255.255.224 | N/A |  |
| Branch-GW | Gig 0/0.2 | 192.168.2.1 | 255.255.255.0 | N/A |  |
|  | Gig 0/0.3 | 192.168.3.1 | 255.255.255.0 | N/A |  |
|  | Gig 0/1 | 64.100.1.2 | 255.255.255.224 | N/A |  |
| Wireless Home Router | G0/1 | 64.100.2.2 | 255.255.255.224 | N/A |  |
|  | Router IP | 192.168.10.1 | 255.255.255.128 | N/A |  |

Hosts Address

| PC-number | VLAN ID | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| MIU |  |  |  |  |
| PC-1 | VLAN 110 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-2 | VLAN 110 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-3 | VLAN 120 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-4 | VLAN 25 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-5 | VLAN 25 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-6 | VLAN 140 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-7 | VLAN 150 | 10.0.0.135 | 255.255.255.224 | 10.0.0.129 |
| PC-8 | VLAN 160 | 10.0.0.250 | 255.255.255.240 | 10.0.0.241 |
| PC-9 | VLAN 170 | 10.0.0.200 | 255.255.255.224 | 10.0.0.193 |
| PC-10 | VLAN 180 | 10.0.1.5 | 255.255.255.240 | 10.0.1.1 |
| MIU_ Branch 1 |  |  |  |  |
| PC-11 | VLAN 2 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-12 | VLAN 3 | From DHCP Server | From DHCP Server | From DHCP Server |
| Wireless Home Network |  |  |  |  |
| Laptop |  |  |  |  |
| Tablet |  |  |  |  |
| Smartphone |  |  |  |  |
| Servers |  |  |  |  |
| DHCP Server |  | 10.0.1.19 | 255.255.255.240 | 10.0.1.18 |
| Email Server |  | 10.0.1.22 | 255.255.255.240 | 10.0.1.18 |
| Web server |  | 10.0.1.21 | 255.255.255.240 | 10.0.1.18 |
| DNS server |  | 10.0.1.20 | 255.255.255.240 | 10.0.1.18 |
| NTP and Syslog server |  | 10.0.1.23 | 255.255.255.240 | 10.0.1.18 |

a.     Configure all Devices with the following:

3)     Set the minimum password length to 10 characters.

4)     Use the MIU1234567 password for Console access password.

5)     Use the CSC1234567 secret password.

Branch -> email  & web error

VPN check error